Exercise 1: nslookup (Not Marked)

Use the nslookup command from the "Tools of the Trade" and answer the following questions:

- Which is the IP address of the website www.telstra.com.au? In your opinion, what is the reason for having several IP addresses as an output? You can ignore any IPv6 addresses.

- Find out the name of the IP address 127.0.0.1. What is special about this IP address?

Exercise 2: Use ping to test host reachability (2 marks. 0.2 per each host)

Are the following hosts reachable from your machine by using ping:

- www.google.co.uk

- www.columbia.edu

- www.wikipedia.org

- ec.ho

- hhh.gs

- defence.gov.au

- yes.no

- one.one.one.one

- theguardian.com

- xn--i-7iq.ws

If you observe that some hosts are unreachable, can you explain why? Check if the addresses unreachable by the ping command are reachable from the Web browser.

Exercise 3: Use traceroute to understand the network topology (4 marks)

Note: Include all traceroute outputs in your report.

- Run traceroute on your machine to usi.ch (NOT www.usi.ch ) .

  - How many routers are there between your workstation and usi.ch? How many routers along the path are part of the UNSW network?

  - Which router is the first router outside of Australia?

  - Which router is the first router in Europe/UK? HINT: compare the round trip times from your machine to the routers. You might also find some router names informative and/or looking at network maps (e.g. for AARNeT/ www.submarinecablemap.com ).

- Run a traceroute from your machine to the following destinations: (i) jhu.edu (ii) usp.br and (iii) ed.ac.uk .

  - At which router do the paths from your machine to these three destinations diverge (i.e. which is the last router they have in common)? Find out further details about this router. HINT: You can learn more about a router by running the Whois command: whois router-IP-address.

  - Is the number of hops on each path proportional to the physical distance? HINT: You may be able to find out the geographical location of a server using the following tool - https://iplocation.io/ .

- Several servers are distributed worldwide to provide a web interface from which you can perform a traceroute to any other host on the Internet. Here are two examples: (i) https://www.net.princeton.edu/traceroute.html and (ii) www.as13030.net/traceroute.php .

  - Run a traceroute from both these servers towards your machine and in the reverse direction (from your machine to these servers - do not include the full URL while doing this, e.g. just " www.net.princeton.edu ". You may also try other traceroute servers from the list at www.traceroute.org . What are the IP addresses of the two servers that you have chosen?

  - Does the reverse path go through the same routers as the forward path?

  - If you observe common routers between the forward and the reverse path, do you also observe the same IP addresses? Why or why not?

IMPORTANT : (1) When running this test on your machine connected to UniWide, the reverse traceroute fails because of the 10.x.x.x IP address assigned to your machine is a private IP address (i.e. it is behind a NAT) and thus not publicly routable. So, make sure you conduct the above experiment through VLAB. (2) Feel free to terminate the traceroute if you start receiving output with multiple " * * * " responses or if you can confirm that the traceroute messages have reached the destination network.

Exercise 4: Use ping to gain insights into network performance (4 marks)

Note: Include all graphs in your report. You need to run the scripts (runping.sh and plot.sh) when you are physically using a lab machine or connected to a CSE server/lab machine using VLAB / VNC client. You need to ensure gnuplot and ps2pdf are available on your system if you plan to do this exercise on your machine.

We now use the ping utility to investigate network delay and its implications on network performance. In particular, we will analyze the dependency of packet size and delay.

There is a shell script runping.sh , provided that you can use it instead of running many pings with different packet sizes by hand. After downloading this script on your machine, make sure you can execute it. If not, you must execute the following command in the command line: chmod u+x runping.sh . To run the ping traces, you may use the runping.sh script as follows: ./runping.sh www.abc.net (or whatever other destination you want to ping). It will automatically run ping for different packet sizes, with 50 ping packets per size (-c 50). This script will take a few minutes to finish since ping is sent once per second (-i 1). Additional options are enabled to use IPv4 only (-4) and not lookup symbolic names for host addresses (-n). This script only executes the commands:

$ ping -4 -n -c 50 -i 1 -s 22 www.abc.net > www.abc.net -p50

...

$ ping -4 -n -c 50 -i 1 -s 1472 www.abc.net > www.abc.net -p1500

and writes the output of the pings to the corresponding files.

Use this script for the following destinations:

- cdu.edu.au (Charles Darwin University, Darwin, Australia)

- usp.br ( Universidade de São Paulo (USP),Sao Paulo,Brazil)

- ed.ac.uk (The University of Edinburgh - Edinburgh, Scotland, UK)

Alternatively, use kcl.ac.uk (King's College, London, UK) / columbia.edu ( Columbia University, New York, USA) if any of the above destinations are down.

In other words, execute the following commands:

$ ./runping.sh cdu.edu.au

$ ./runping.sh usp.br

$ ./runping.sh ed.ac.uk

Note that all delay values reported are in milliseconds (ms) and reflect the round trip time (RTT) between your host and the destinations.

If you cannot execute runping.sh, then fix the permissions by running the following command in the command line:

$ chmod u+x runping.sh

When the runping.sh script is finished for all destinations, you can plot the results using another provided script, plot.sh , as follows:

$ ./plot.sh cdu.edu.au-p*

$ ./plot.sh usp.br-p*

$ ./plot.sh ed.ac.uk-p*

If you cannot execute plot.sh, then fix the permissions by running the following command in the command line:

$ chmod u+x plot.sh

The script plot.sh will produce the following files: destination_delay.pdf , destination_scatter.pdf, and destination_avg.txt for each destination (e.g., for cdu.edu.au we have cdu.edu.au_delay.pdf and cdu.edu.au_scatter.pdf and cdu.edu.au_avg.txt ).

The graph destination_delay.pdf shows how delay varies over time (different colours correspond to different packet sizes), and destination_scatter.pdf shows delay vs. packet size as a scatter plot. destination_avg.txt contains the average (2nd column) and minimum (3rd column) delay values corresponding to each packet size (1st column).

- For each location, ﬁnd the (approximate) physical distance from UNSW . You can use a site like Distance Calculator , Google Maps , or whatever you prefer to take this measurement. Then, compute the shortest possible time T for a packet from UNSW to reach that location. You should assume that the packet moves (i.e. propagates) at the speed of light, 3 x 10^8 m/s. Note that the shortest possible time will be the distance divided by the propagation speed.

- Plot a graph where the x-axis represents the distance to each city (i.e. Darwin, Sao Paulo and Edinburgh). The y-axis represents the ratio between the minimum delay (i.e. RTT) measured by the ping program (select the values for 50-byte packets) and the shortest possible time T to reach that city from UNSW. (Note that the y-values are no smaller than 2 since it takes at least 2*T time for any packet to reach the destination from UNSW and return).

You can also use the provided generate_plot.py to generate the plot. Download (to Vlab or personal machines with Python 3 installed). Open the generate_plot.py and uncomment the designated lists, and replace them with the actual values.

3. Can you think of at least two reasons why the y-axis values you plot are greater than 2?

4. Is the delay to the destinations constant, or does it vary over time? Explain why.

5. The measured delay (i.e., the delay you can see in the graphs) comprises propagation, transmission, processing, and queuing delays. Which of these delays depend on the packet size and which do not?